WB-0FC-2N/O
2.4 Ghz 300 Mbps 1 KM Wireless Bridge

The wireless bridge can be applied in industries, such as wireless video security, railway, transportation, power and other industries, wireless video/data transmission, wireless coverage, carrier wireless backbone network construction, Wi-Fi wireless coverage, rural information construction, and wireless network coverage. The long-distance transmission of the device can guarantee multi-channel HD video transmission.

- Up to 1 km wireless transmission distance

- 300 Mbps 802.11n wireless bridge

- Built-in 7dBi 2 × 2 MIMO antenna

- Visualized topology management

- Real-Time alarm push technology

| - Specification |
| --- |
| Wireless Parameter |  |
| Max. Transmission Rate of Air Interface | ≤ 300 Mbps |
| Wireless Standard | IEEE 802.11b/g/n (2 × 2 MIMO 300 Mbps) |
| Working Frequency Band | 2400 MHz ~ 2483.5 MHz |
| Receiving Sensitivity | -87±2 dBm @HT40 MCS0 |
| Antenna Gain | 7 dBi |
| Antenna Angle | (H) Beamwidth [3 dB] 35° (V) Beamwidth [3 dB] 35° |
| Transmitting Power | 23 dBm @11n MCS0 (Note: Transmitting power may vary with local regulations.) |
| Throughput Rate | PTP: 90 Mbps/0.2 KM(HT40), 75 Mbps/0.5 KM(HT40), 40 Mbps/1 KM(HT40) PTMP(1 to 4): 20 Mbps/1 KM(HT40) |
| Hardware Parameter |  |
| Network Interface | 2 × 10/100 Mbps RJ45 ports |
| LED Indicator | Power indicator, LAN indicator, Signal intensity indicator |
| Reset | Hardware resetting button |
| Power Supply | 12-24 VDC Passive PoE |
| Software Function |  |
| Application Function | TDMA, DFS, TPC, IFM, IPM |
| Channel Width | 10/20/40 MHz |
| Channel Selection | Auto/manual selection |
| Network Protocol | NTP (network time synchronization), SADP (auto search for IP address), HTTPS (web management), SSH (debugging) |
| Security Mode | WPA2-PSK |
| Security Mechanism | Wireless network name hidden |
| Management Method | Web |
| Upgrade | Web |
| System Log | Syslog, Control center information |
| General |  |
| Product Dimensions (W × H × D) | 81 mm × 52 mm × 165 mm (3.62'' × 2.28'' × 7.36'') |
| Package Dimensions (W × H × D) | 132 mm × 228 mm × 91 mm (5.19" × 8.97" × 3.58") |
| Net Weight | 0.19 kg (0.42 lb) |
| Gross Weight | 0.49 kg (1.08 lb) |
| Packing List | Wireless Bridge × 1, Power Adapter (12 VDC, 1 A) × 1, 10/100 Mbps PoE Injector × 1, Binding Tape × 2, Quick Start Guide ×1, Regulatory Compliance and Safety Information × 1 |

| Packing Method | Single unit packing |
| --- | --- |
| Working Temperature | -30 °C to 70 °C (-22 °F to 158 °F) |
| Working Humidity | 5% to 95% (no condensation) |
| Storage Temperature | -40 °C to 80 °C (-40 °F to 176 °F) |
| Storage Humidity | 5% to 95% (no condensation) |
| Installation Mode | Pole Mounting |
| Ingress Protection | IP55 |
| Power Consumption | 1.8 W for typical (1-ch 2MP IPC), Max. 4.5 W |
| Approval |  |
| EMC | CE-EMC (EN 55032: 2015+A11: 2020, EN IEC 61000-3-2: 2019, EN 61000-3-3: 2013+A1: 2019, EN 50130-4: 2011+A1: 2014, EN 55035: 2017+A11: 2020) |
| Safety | CB (AMD1:2009, AMD2:2013, IEC 62368-1: 2014 (Second Edition)), CE-LVD (EN 62368-1: 2014+A11: 2017), UL (UL 60950-1) |
| Chemistry | CE-RoHS (2011/65/EU), WEEE (2012/19/EU), Reach (Regulation (EC) No.1907/2006) |
| Radio | EN 301 893, EN 300 440, EN 62311, EN 301 489-3, EN 301 489-17 |

- Typical Application

- Physical Interface

| No. | Interface/Button | Description |
| --- | --- | --- |
| 1 | Reset | Long press for 4 seconds to restore to factory settings. |
| 2 | PoE/LAN 1 | The 10/100 Mbps adaptive RJ45 port, also can be used as a PoE port. |
| 3 | LAN 2 | The 10/100 Mbps adaptive RJ45 port. |
| 4/5 | Network Cable Slot | Reserved for easy placement of the network cables. |

- Available Model WB-0FC-2N/O

- Dimension

Unit: mm